# Editing Equinox Screen File Packages

NOTE:  All files discussed in this documentation can be found on the ftp site, ftp.mtxeps.com.  Contact the Helpdesk at 949-614-1616 or email DL-Connected Services Support@Retalix.com if you need a login to the ftp site.

# Option 1:  Use Retalix Global Payments Generic Screen File Package, but replace the Retalix logo with the customer’s logo.

- Create logo file in the proper format and size for the specific terminal using the graphics editor of your choice.  Graphics work best when in a landscape format, especially for the L4250 and L5200 terminals.

  - L5300 = .jpg = 640x160 pixels = color

  - L5200 = .jpg = 480x80 pixels = color

  - L4150 = .bmp = 320x80 pixels = color

  - L4100 = .bmp = 320x80 pixels = color

  - L4250 = .bmp = 220x30 pixels = monochrome

Default Logo used in 4250 screen file packages

Default Logo used in all other screen file packages

- For the 4xxx series of terminals you will need to convert the .bmp file to an .img file.

  - Download the Hypercom Image Converter application from our FTP site. (Tools\Screen File Creation\Hypercom\Hypercom 4100 Image Converter)

  - After installing the Image Converter application, run conv2nuimg_2.exe.

  - The Source Image will be the new graphic you created/edited.

  - The Destination Image should be named logo.img.

  - Press Convert for the file to be processed.

- For the 5xxx series of terminals you just use the straight .jpg file that you created.

- Procedures:

  - Get the Windows properties for the .img/.jpg file that you created and make note of its file size in bytes:

  - Locate the standard Hypercom screen file zip package for the terminal you are using.  (Hypercom 4100.zip, Hypercom 4150.zip, Hypercom 4250.zip, Hypercom 5300.zip, Hypercom 5200.zip, etc.)

  - Replace the default logo.img or logo.jpg file from the zip file with the new logo file you created.

  - Remove the packinglist file from the zip package and update it.

    - Modify the file size on the logo file line to match the size of the new file.

    - Increase the packing list number on the top line of the file by one.

    - Put the updated packinglist file back into the zip package.  (Overwrite the existing if asked.)

  - Save the new screen file zip package with a file name that is more descriptive or notes the customer name or graphic in use.  This new file will be the updated screen file referenced in the lane configurations and setups.

# Option 2:  Use Formbuilder to modify the Retalix Global Payments Generic Screen File Package.

- Obtain and install the current version of Formbuilder from Equinox.  The latest software being used by Retalix can be pulled from the ftp site under Tools\Screen File Creation\Hypercom\Hypercom Form Builder, however a Sentinel USB dongle must be obtained from Equinox in order to use the application.

  - Formbuilder 6.22-rc11 is the version being used by Retalix at the time of this documentation and is capable of creating forms for all the L4xxx as well as the L5300 2.0 and the L5300 and L52-rc00 3.0 terminals. Future released terminals and code may required a newer version of Formbuilder as well.

  - The USB driver for the Sentinel hasp key is also located in the Form Builder directory of the ftp site.  This will need to be installed on the pc where the Formbuilder application will be used.  Ver 7.5.7 is required for Formbuilder 6.22.

- Obtain the Screen File Source Code packages from the ftp site. (Tools\Screen File Creation\Hypercom\Source Code)

  - There will be two Source Code Project packages for each terminal type.

    - Non-Signed Screens project – These screens can be manipulated by the retailer/dealer without Equinox intervention.

    - Signed Screens project– These screens are controlled by the PCI PED requirements and must be compiled and signed into a TCMS file by Equinox.

      - These screens must be created/modified and then the entire project sent to Equinox for review and signing.  NOTE:  Their Security team only meets to review and sign screens on Wednesdays.

      - The .TCMS file returned from Equinox must then be imported into the Non-Signed screen project and compiled for the final package.

  - Unzip the default source code project packages into the c:\Program Files\Equinox\FormBuilder\Projects directory.

  - Launch the FormBuilder

- Using FormBuilder

  - Open a Project

    - Existing Project

      - Select File and then Open Project from the main menu.

      - Browse to the c:\Program Files\Equinox\FormBuilder\Projects directory and the directory where you unzip the project you want to work on.

      - Select the *.vsp file, and click on Open.

    - New Project

      - Select File and then New Project from the main menu.

      - You can build a new project from scratch using the New Project radio button.

        - Select which terminal platform you are created the project for by clicking on the appropriate terminal icon.

        - Enter a new name for the project in the Project Name field.

        - Use the … browse button to select a location where you want the project folder to be created.

        - Select OK.

      - You can also build a new project starting from an existing project using the Import Project radio button.

        - Select the location of the existing project that you want to import from in the first Location field.  Use the … browse button to the right of the field to find the project.

        - Enter a new name for the project in the Project Name field.

        - Use the … browse button to the right of the second Location field to select a location where you want the project folder to be created.

        - Select OK.  This will create a new project using the new name provided and import all of the screens from the existing screen set, using the same terminal platform as the original.

  - Overview

    - Down the left side of the screen will be a list of all the Screen names.  The top of that list is the Project name with the terminal Platform being used in parenthesis.  (L4100, L4250, L4150, L4150 PCI 2.x, L5300, L5300_PCI3 or L5200_PCI3)

    - The Messages section at the bottom updates as you open, update or compile a project.

    - Double click on the Screen Name at the left and the screen will be displayed at the right.

    - The left side also opens up to show the controls, buttons, image files, and SubScreens on the screen.

    - Click the Preview button at the top of the screen to see what the screen will look like on the terminal.  (Include the screen with it’s subscreen.)  Click Exit Preview to go back to the editing screen.

    - The Toolbox appears when a screen is opened.  This allows Buttons, text boxes, picture boxes , Video boxes and other controls to be added to the screen.  <<See the FormBuilder User Guide.pdf – Chapter 2 for details on how to implement each of the toolbox controls.>>

    - Right click on any control on the screen and select Properties to edit the properties of that control, or double click the control in the list on the left hand side of the screen.

    - Additional Files

      - You can import files and fonts to be used with the screen file package.

        - Right click on the Additional files line in the screen name list at the left side of the window.

        - Select Import File, Import Font or Import Signed RPM

      - The following are the standard added files that should be included with the screen file package.

        - wicproms.cfg – needed for SmartWIC transactions.

        - Signed Screens TCMS file.

          - Standard format for default files is TTRMMYY.tcms

            - TT = terminal abbreviation

            - R = R for Retalix or X for PCI 3.0

            - MMYY = Month and Year the screens were created and signed.

            - Example = 53R0612.tcms

              - These are for the L5300 2.0 terminals and were created/signed in June of 2012.)

          - If the screens from the Signed Screens package are added to the normal screen file project, without being signed and compiled into a tcms file by Equinox, those screens will not fully display or operate properly when loaded into the production terminals.

        - White List TCMS file

          - A whitelist is needed in the new PCI 3.0 environment in order to run full Point to Point.  This is an xml file that contains the BIN ranges of the cards that can be provided to OpenEPS unencrypted.  It is imported into a screen package that contains one blank screen and then has to be signed and compiled into a TCMS file by Equinox, just like the signed screen package.

          - If the Point to Point OpenEPS dll is being used and Point to Point is desired then the Whitelist is not necessary.  No cards will be decrypted within the store.

          - If the Point to Point OpenEPS dll is being used and Point to Point is desired by the retailer, however there are certain cards that they still want to see the track data from, then a whitelist needs to be created that includes those card ranges.

          - If the non Point to Point OpenEPS dll is being used then a black holed whitelist needs to be included so that the terminal will provide OpenEPS the decrypted card data.

            - Black holed whitelists will be included with the generic default screen file sets for all terminals.  There are currently two files in use, depending on the PCI version being used.  WL_ALL_PCI2.tcms and WL_ALL_PCI3.tcms

        - FPE TCMS file.

          - This is required in the 5xxx version terminals, but not in the 4xxx terminals.  The file should be compiled into the complete screen file package for loading.

          - Should be imported using Import Signed RPM.

          - The RPM file name = the SCAT TCMS file name.

          - Message = the SCAT Version name

          - Reboot after install = Soft

          - Cancel Installation on Failure = No

  - Compiling the Project.

    - Once you have all the modifications to the project and you are ready to build the screen file package.

    - Select the Compile Project button at the top in about the middle of the screen.

    - Each screen should scroll but in the Messages window at the bottom of the screen.  It should say Project compile succeeded when done and then copy any additional files assigned to the project into the output directory.

    - The output directory is a \dl\ located within the project directory you opened and compiled.  Zip up the contents of the dl directory and give it a descriptive name.  Do not zip the directory itself, just the files.  This is your new screen file package.

  - General items of interest.

    - Flag to erase existing files from terminal before loading new.

      - Select Project and then Settings from the main menu.

      - Select Erase previous packinglist from the Legacy Compatibility tab.

        - This will clear the existing packing list content from the terminal before beginning to load a new packinglist.  This will prevent screens and files from remaining in the terminal that are no longer needed.

    - Flag to determine where Finger or Stylus can be used on a screen.

      - Right click the Screen Name on the left side of the window and select Properties from the menu to display the Screen Properties window.

      - Under the Touch Screen Mode section at the bottom right.  Select whether you want Stylus + Finger, Stylus only or Finger only to work on that particular screen.

      - Default should be Stylus + Finger.

    - Flag to turn RFID on and Off.

      - Open the SWIPEFRM screen (or any screen that contains the MSR control.

      - Double click the MSR control (or right click and select Properties)

      - On the General tab check or uncheck the Enable ISO 1443 A/B card reader depending on if you want the reader turned on or off.

    - Button controls

      - For a button control on the default screens you can change the ForeColor, BackColor, Font, X, Y, Width and Height fields.  (Make sure to preview how the screen will look with the subscreens so that you don’t overlap visible items.)

      - If the Caption field begins with @ like the @B1 shown above, then do not change it.  This allows OpenEPS to fill in the caption of the button from what has been configured within WinEPS or ServerEPS.  If the caption is a static text, then you can modify it as you wish.

      - Control ID, VrtKbdChar and Return Key should not be changed.  These determine the buttons function within the interface to OpenEPS.

    - Image controls

      - The Image control can resized and/or moved to any place on the screen.

      - Make sure to preview how the screen will look with the subscreens so that you don’t overlap visible items.

    - Data Entry controls

      - You can set the Field Type being used for the Data Entry field you are creating. (Numeric Entry, Alphanumeric, Amount, Masked Entry, Date, PAN, SSN, Phone, etc.)

      - You can set the MaxLength and MinLength that will be allowed for the entry.

      - You can also set whether to mask the entry (Masked) as it is entered and how long of a timeout to give before each character entered is masked (Masking Timeout.)

      - Most screens containing the Data Entry control will have to be Signed by Equinox.

    - Video Controls

      - See page 2-22 of the FormBuilder User Guide.pdf for directions on how to implement the use of Videos with the terminals.

        - For the L5xxx, this video file must be an MPEG-4 file with a .mp4 extension.  For the L4150, this video file must be an MPEG-1 or MPEG-2 file.  Other versions of MPEG files are not supported.

# Screen Files Listings

|  |  | = screens contain the logo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | = screens contain scrolling lines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | = screens contain logo and scrolling lines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L4250 |  |  |  |  |  |  |  |  | L41xx/L5200/L5300 |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-Signed Screens |  |  |  |  | Signed Screens |  |  |  | Non-Signed Screens |  |  |  |  |  |  |  | Signed Screens |  |  |  |  |
| Screen Name | Sub-Screen |  |  | Screen Name |  | Sub-Screen |  |  |  |  | Screen Name |  |  | Sub-Screen |  |  |  | Screen Name |  | Sub-Screen |  |  |
| 1KEYFRM | bgglob |  |  | AMNTFRM |  |  |  |  |  |  | 1KEYFRM |  |  | bgdad |  |  |  | AMOUNTFRM |  |  |  |  |
| BADSWIPE | bgglob |  |  | AMOUNTFRM |  |  |  |  |  |  | CLOSEDFRM |  |  | bgdad |  |  |  | AMTFRM |  |  |  |  |
| CBYNFRM | bgglob |  |  | FLEETDRVID |  |  |  |  |  |  | PROMPTS |  |  | bgdad |  |  |  | FLEETDRVID |  |  |  |  |
| CLOSEDFRM | bgglob |  |  | FLEETODOM |  |  |  |  |  |  | SWIPEFRM |  |  | bgdad |  |  |  | FLEETODOM |  |  |  |  |
| MESSAGEKEY | bgglob |  |  | FLEETVEHID |  |  |  |  |  |  | WICCANFRM |  |  | bgdad |  |  |  | FLEETVEHID |  |  |  |  |
| PROMPTS | bgglob |  |  | NUMCVV2 |  |  |  |  |  |  | WICINPFRM |  |  | bgdad |  |  |  | NUMCVV2 |  |  |  |  |
| PURBALFRM | bgglob |  |  | NUMEXPDATE |  |  |  |  |  |  | WICLOCFRM |  |  | bgdad |  |  |  | NUMEXPDATE |  |  |  |  |
| SELECTEBT | bgglob |  |  | NUMPAN |  |  |  |  |  |  | WICRETFRM |  |  | bgdad |  |  |  | NUMPAN |  |  |  |  |
| SOFTKEY03 | bgglob |  |  | NUMRFRM |  |  |  |  |  |  | WICSTFRM |  |  | bgdad |  |  |  | NUMRFRM |  |  |  |  |
| SOFTKEY08 | bgglob |  |  | PHONENUMFRM |  |  |  |  |  |  | WICENDFRM |  |  | bgdad2 |  |  |  | PHONENUMFRM |  |  |  |  |
| SOFTKEY09 | bgglob |  |  | PINFRM |  |  |  |  |  |  | WICREMFRM |  |  | bgdad2 |  |  |  | PINFRM |  |  |  |  |
| SOFTKEY17 | bgglob |  |  | PINFRM9 |  |  |  |  |  |  | WICRXFRM |  |  | bgdad2 |  |  |  | PINFRM9 |  |  |  |  |
| SOFTKEY18 | bgglob |  |  | PINFRMA |  |  |  |  |  |  | WICWTFRM |  |  | bgdad2 |  |  |  | PINFRMA |  |  |  |  |
| SOFTKEY19 | bgglob |  |  | PINFRMB |  |  |  |  |  |  | CBBUTTFRM |  |  | bgdnad |  |  |  | PINFRMB |  |  |  |  |
| SWIPEFRM | bgglob |  |  | PINFRMC |  |  |  |  |  |  | CBYNFRM |  |  | bgdnad |  |  |  | PINFRMC |  |  |  |  |
| WAITFRM | bgglob |  |  | PINFWCR |  |  |  |  |  |  | LANGFRM |  |  | bgdnad |  |  |  | PINFWCR |  |  |  |  |
| WICCANFRM | bgglob |  |  | PINFWCR9 |  |  |  |  |  |  | MESSAGEKEY |  |  | bgdnad |  |  |  | PINFWCR9 |  |  |  |  |
| WICENDFRM | bgglob |  |  | PINFWCRA |  |  |  |  |  |  | PURBALFRM |  |  | bgdnad |  |  |  | PINFWCRA |  |  |  |  |
| WICINPFRM | bgglob |  |  | PINFWCRB |  |  |  |  |  |  | SELECTEBT |  |  | bgdnad |  |  |  | PINFWCRB |  |  |  |  |
| WICLOCFRM | bgglob |  |  | PINFWCRC |  |  |  |  |  |  | SIGNATURE |  |  | bgdnad |  |  |  | PINFWCRC |  |  |  |  |
| WICREMFRM | bgglob |  |  | WICPINFRM |  |  |  |  |  |  | SOFTKEY03 |  |  | bgdnad |  |  |  | SLIDENUM |  |  |  |  |
| WICRETFFRM | bgglob |  |  |  |  |  |  |  |  |  | SOFTKEY07 |  |  | bgdnad |  |  |  | VIRTUALPIN |  |  |  |  |
| WICRXFRM | bgglob |  |  |  |  |  |  |  |  |  | SOFTKEY08 |  |  | bgdnad |  |  |  | WICPINFRM |  |  |  |  |
| WICSTFRM | bgglob |  |  |  |  |  |  |  |  |  | SOFTKEY09 |  |  | bgdnad |  |  |  |  |  |  |  |  |
| WICWTFRM | bgglob |  |  |  |  |  |  |  |  |  | SOFTKEY17 |  |  | bgdnad |  |  |  |  |  |  |  |  |
| YNFRM | bgglob |  |  |  |  |  |  |  |  |  | SOFTKEY18 |  |  | bgdnad |  |  |  |  |  |  |  |  |
| SIGNATURE | bgglobhi |  |  |  |  |  |  |  |  |  | SOFTKEY19 |  |  | bgdnad |  |  |  |  |  |  |  |  |
| SIGNATURE2 | bggloblo |  |  |  |  |  |  |  |  |  | YNFRM |  |  | bgdnad |  |  |  |  |  |  |  |  |
| CBBUTTFRM | bgnad |  |  |  |  |  |  |  |  |  | SIGNATURE2 |  |  | bgdnad2 |  |  |  |  |  |  |  |  |
| SOFTKEY20 | bgnad2 |  | <--No subscreen in scrolling package |  |  |  |  |  |  |  | GETTENDFRM |  |  | glob |  |  |  |  |  |  |  |  |
| SOFTKEY27 | bgnad2 |  | <--No subscreen in scrolling package |  |  |  |  |  |  |  | SOFTKEY20 |  |  | glob |  |  |  |  |  |  |  |  |
| SOFTKEY28 | bgnad2 |  | <--No subscreen in scrolling package |  |  |  |  |  |  |  | SOFTKEY27 |  |  | glob |  |  |  |  |  |  |  |  |
| GETTENDFRM |  |  |  |  |  |  |  |  |  |  | SOFTKEY28 |  |  | glob |  |  |  |  |  |  |  |  |
| LANGFRM |  |  |  |  |  |  |  |  |  |  | BADSWIPE |  |  |  |  |  |  |  |  |  |  |  |
| SOFTKEY13 |  |  |  |  |  |  |  |  |  |  | SOFTKEY13 |  |  |  |  |  |  |  |  |  |  |  |
| SOFTKEY14 |  |  |  |  |  |  |  |  |  |  | SOFTKEY14 |  |  |  |  |  |  |  |  |  |  |  |
| SOFTKEY37 |  |  |  |  |  |  |  |  |  |  | SOFTKEY37 |  |  |  |  |  |  |  |  |  |  |  |
| SOFTKEY38 |  |  |  |  |  |  |  |  |  |  | SOFTKEY38 |  |  |  |  |  |  |  |  |  |  |  |
| SOFTKEY39 |  |  |  |  |  |  |  |  |  |  | SOFTKEY39 |  |  |  |  |  |  |  |  |  |  |  |